K32	Panel cubicles/ duct and wall linings/ screens

120/01		N13/ PANEL CUBICLES - UNFRAMED

- Manufacturer:. Mode Washroom Ltd, Unit 29 Park Farm Drive, Allestree, Derby, DE22 2QQ  Tel: 07896424563 – James Purrett

Product name: Lux standard height range – Flush fronted, discrete fitting and floating appearance gives a clean, sleek architectural appearance

Board/panel type: 1300kg/m³ High performance density solid grade laminate (SGL) with decorative face each side (Black core).

Thickness: 13mm nominal.

Edge treatment: mode safety edge

Overall depth: 2000mm max from rear wall or face of ducting.

Floor Clearance: insert floor clearance here

- Range height: 2100mm from FFL

Headrail: Aluminium profile, cubicles to finish 100mm below ceiling height.

Hinges: Stainless steel sprung hinge, return to closed

Pedestals: Aluminium with adjustment

Indicator bolt and keep: Stainless steel indicator bolt with circular face plate and emergency release facility

Wall fixings: satin anodised aluminium brackets

Board/panel Colour/Finish: From modes standard colour range

Optional Accessories: See N13 for options

Unity SGL floor clearance

- 100mm floor clearance

- 20mm floor clearance